# IVP Assignment - Solutions

Name: ____________________        Roll_No: ____________________

Convention: pixel positions are (row, column), 1-based.

# Question 1: Connected components

Binary image I:

| 1 | 0 | 1 | 0 |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |

Foreground pixels (value 1): (1,1) (1,3) (2,2) (2,4) (3,1) (3,3) (4,2) (4,4) - 8 pixels in total.

## (a) 4-connectivity

Theory: with 4-connectivity, pixel p(x,y) is adjacent only to its up, down, left and right neighbours: (x-1,y), (x+1,y), (x,y-1), (x,y+1). Diagonal neighbours do not count.

Take (1,1). Its 4-neighbours are (1,2)=0 and (2,1)=0. It has no foreground neighbour, so it is a component on its own. Component 1.

(1,3): neighbours (1,2)=0, (1,4)=0, (2,3)=0. Isolated. Component 2.

(2,2): neighbours (1,2)=0, (3,2)=0, (2,1)=0, (2,3)=0. Isolated. Component 3.

(2,4): neighbours (1,4)=0, (3,4)=0, (2,3)=0. Isolated. Component 4.

(3,1): neighbours (2,1)=0, (4,1)=0, (3,2)=0. Isolated. Component 5.

(3,3): neighbours (2,3)=0, (4,3)=0, (3,2)=0, (3,4)=0. Isolated. Component 6.

(4,2): neighbours (3,2)=0, (4,1)=0, (4,3)=0. Isolated. Component 7.

(4,4): neighbours (3,4)=0, (4,3)=0. Isolated. Component 8.

Reason: the 1s sit in a checkerboard pattern, so every horizontal and vertical neighbour of a 1 is a 0.

Answer (a): 8 connected components.

## (b) 8-connectivity

Theory: with 8-connectivity, diagonal neighbours also count, so each pixel has up to 8 neighbours.

Start a flood fill at (1,1). Its diagonal neighbour (2,2)=1, so add (2,2).

From (2,2) the diagonal neighbours are (1,1), (1,3), (3,1), (3,3). All are 1, so add (1,3), (3,1), (3,3).

From (1,3): diagonal neighbour (2,4)=1, add (2,4). From (3,3): diagonal neighbours (2,4), (4,2), (4,4) are all 1, so add (4,2) and (4,4).

From (3,1): diagonal neighbour (4,2) is already in the set.

All 8 foreground pixels have been visited from a single starting point.

Visit order: (1,1) -> (2,2) -> (1,3), (3,1), (3,3) -> (2,4), (4,2), (4,4).

Answer (b): 1 connected component.

Conclusion: the same image has 8 components under 4-connectivity but 1 under 8-connectivity, so the choice of connectivity changes the result a lot.

# Question 2: Averaging filter and high-boost filtering

Neighbourhood f:

| 20 | 25 | 30 |
| --- | --- | --- |
| 25 | 100 | 35 |
| 30 | 35 | 40 |

## (a) Blurred centre pixel

The 3x3 averaging mask has every coefficient 1/9:

| 1/9 | 1/9 | 1/9 |
| --- | --- | --- |
| 1/9 | 1/9 | 1/9 |
| 1/9 | 1/9 | 1/9 |

Add all nine pixels: 20+25+30 = 75 (row 1); 25+100+35 = 160 (row 2); 30+35+40 = 105 (row 3).

Total = 75 + 160 + 105 = 340.

f_blur = 340 / 9 = 37.78.

Answer (a): the blurred centre pixel is 37.78 (about 38 if rounded to an integer).

## (b) High-boost filtering with A = 2.5

Formula given: g = A*f - f_blur. This is the same as the usual form g = (A-1)*f + (f - f_blur), where f - f_blur is the unsharp mask, so the sharpening is added back to the (A-1) times original.

Centre pixel f = 100, A = 2.5, f_blur = 37.78.

A*f = 2.5 x 100 = 250.

g = 250 - 37.78 = 212.22.

Check with the other form: (A-1)f + (f - f_blur) = 1.5(100) + (100 - 37.78) = 150 + 62.22 = 212.22. Both forms agree.

212.22 lies between 0 and 255, so clipping is not needed. The output is about 212.

Answer (b): g = 212.22 (about 212). The centre pixel brightens from 100 to 212, because it stands out from its neighbours and the filter boosts that difference.

# Question 3: Hit-or-Miss transformation with a non-symmetric structuring element

Image A (5x5):

| 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 |

Template (the centre is the middle cell, 1 = must be foreground, 0 = must be background):

| 1 | 1 | 0 |
| --- | --- | --- |
| 1 | 1 | 0 |
| 0 | 0 | 0 |

The template is not symmetric (not the same under rotation or reflection), so the result depends on its orientation.

## Step 1: Split the template into two structuring elements

Offsets are written as (row offset, column offset) from the centre.

B1 (foreground SE, must match 1): (-1,-1), (-1,0), (0,-1), (0,0).

B2 (background SE, must match 0): (-1,+1), (0,+1), (+1,-1), (+1,0), (+1,+1).

B1 and B2 do not overlap. B1 ∩ B2 = empty, which is required for the transform to make sense.

B1 as a mask:

| 1 | 1 | 0 |
| --- | --- | --- |
| 1 | 1 | 0 |
| 0 | 0 | 0 |

(1 = element belongs to B1, 0 = not part of B1).

B2 as a mask (1 = element belongs to B2):

| 0 | 0 | 1 |
| --- | --- | --- |
| 0 | 0 | 1 |
| 1 | 1 | 1 |

## Step 2: Formula

A ⊛ B = (A ⊖ B1) ∩ (A^c ⊖ B2)

A pixel is in the output only if the pattern B1 fits inside the foreground of A, and the pattern B2 fits inside the background of A, both with the same centre.

## Step 3: Erosion A ⊖ B1

A pixel (r,c) survives erosion only if A(r,c), A(r-1,c), A(r,c-1) and A(r-1,c-1) are all 1. Only the 1-pixels of A can be a centre, so test each:

| Centre | A(r,c) | A(r-1,c) | A(r,c-1) | A(r-1,c-1) | Fits B1? |
| --- | --- | --- | --- | --- | --- |
| (2,2) | 1 | A(1,2)=0 | A(2,1)=0 | A(1,1)=0 | No |
| (2,3) | 1 | A(1,3)=0 | A(2,2)=1 | A(1,2)=0 | No |
| (3,2) | 1 | A(2,2)=1 | A(3,1)=0 | A(2,1)=0 | No |
| (4,4) | 1 | A(3,4)=0 | A(4,3)=0 | A(3,3)=0 | No |

A ⊖ B1 is therefore a 5x5 matrix of all zeros. B1 requires a full 2x2 block of 1s, and A has none.

## Step 4: Erosion of the complement

A^c is A with 0 and 1 swapped. A^c ⊖ B2 marks the centres where all 5 B2 positions are background in A. Many pixels satisfy this, but since A ⊖ B1 is empty it does not change the final result: the intersection of an empty set with anything is empty.

## Step 5: Result

| 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 |

Answer: no location in the image matches the template, so the Hit-or-Miss output is empty.

## Interpretation

The template detects the bottom-right corner of a filled 2x2 block of 1s with background to the right and below it.

The L-shape at (2,2), (2,3), (3,2) is only three pixels; (3,3) is 0, so it is not a 2x2 block. It is not detected.

The isolated pixel at (4,4) is also not detected, because the template needs three foreground neighbours (up, left, up-left) and (4,4) has none.

Effect of 90 degree rotation: rotating the template by 90 degrees clockwise gives foreground offsets (-1,0), (-1,+1), (0,0), (0,+1), i.e. it detects the bottom-left corner of a 2x2 block. Because the template is non-symmetric, each rotation detects a different corner, so all four rotations are needed to find all corners. In this image none of the rotations match anything, as there is no 2x2 block.

# Question 4: Region growing (4-connectivity)

Image I:

| 20 | 22 | 25 | 80 | 82 |
| --- | --- | --- | --- | --- |
| 21 | 23 | 27 | 78 | 81 |
| 19 | 24 | 50 | 76 | 79 |
| 18 | 22 | 52 | 75 | 77 |
| 17 | 20 | 55 | 73 | 76 |

Seeds: S1 = (2,2) with intensity 23, S2 = (4,4) with intensity 75. Predicate (from the question paper): a pixel p joins the region if |I(p) - I(seed)| <= 8. The seed intensity stays fixed. Neighbours are the 4 pixels up, down, left and right.

## (a) Region from S1 = (2,2), intensity 23

Accepted range: 23 - 8 = 15 to 23 + 8 = 31, so a pixel joins if 15 <= I <= 31.

| Step | Pixel examined (from) | Intensity | ¦I - 23¦ | Accepted? |
| --- | --- | --- | --- | --- |
| 0 | (2,2) seed | 23 | 0 | Yes (start) |
| 1 | (1,2) from (2,2) | 22 | 1 | Yes |
| 2 | (3,2) from (2,2) | 24 | 1 | Yes |
| 3 | (2,1) from (2,2) | 21 | 2 | Yes |
| 4 | (2,3) from (2,2) | 27 | 4 | Yes |
| 5 | (1,1) from (1,2) | 20 | 3 | Yes |
| 6 | (1,3) from (1,2) | 25 | 2 | Yes |
| 7 | (1,4) from (1,3) | 80 | 57 | No |
| 8 | (2,4) from (2,3) | 78 | 55 | No |
| 9 | (3,3) from (2,3) | 50 | 27 | No |
| 10 | (3,1) from (3,2) or (2,1) | 19 | 4 | Yes |
| 11 | (4,2) from (3,2) | 22 | 1 | Yes |
| 12 | (3,3) from (3,2) | 50 | 27 | No (again) |
| 13 | (4,1) from (3,1) | 18 | 5 | Yes |
| 14 | (5,1) from (4,1) | 17 | 6 | Yes |
| 15 | (5,2) from (4,2) | 20 | 3 | Yes |
| 16 | (4,3) from (4,2) | 52 | 29 | No |
| 17 | (5,3) from (5,2) | 55 | 32 | No |

Region S1 = column 1 (all 5 pixels) + column 2 (all 5 pixels) + (1,3) + (2,3) = 12 pixels.

Region map (R = in S1):

| R | R | R | . | . |
| --- | --- | --- | --- | --- |
| R | R | R | . | . |
| R | R | . | . | . |
| R | R | . | . | . |
| R | R | . | . | . |

Answer (a): S1 region = {(1,1),(2,1),(3,1),(4,1),(5,1),(1,2),(2,2),(3,2),(4,2),(5,2),(1,3),(2,3)}. Growth stops because every neighbouring pixel outside has intensity 50 to 80, far outside 15-31.

## (b) Region from S2 = (4,4), intensity 75

Accepted range: 75 - 8 = 67 to 75 + 8 = 83, so a pixel joins if 67 <= I <= 83.

| Step | Pixel examined (from) | Intensity | ¦I - 75¦ | Accepted? |
| --- | --- | --- | --- | --- |
| 0 | (4,4) seed | 75 | 0 | Yes (start) |
| 1 | (3,4) from (4,4) | 76 | 1 | Yes |
| 2 | (5,4) from (4,4) | 73 | 2 | Yes |
| 3 | (4,5) from (4,4) | 77 | 2 | Yes |
| 4 | (4,3) from (4,4) | 52 | 23 | No |
| 5 | (2,4) from (3,4) | 78 | 3 | Yes |
| 6 | (3,5) from (3,4) | 79 | 4 | Yes |
| 7 | (3,3) from (3,4) | 50 | 25 | No |
| 8 | (5,5) from (5,4) or (4,5) | 76 | 1 | Yes |
| 9 | (5,3) from (5,4) | 55 | 20 | No |
| 10 | (1,4) from (2,4) | 80 | 5 | Yes |
| 11 | (2,5) from (2,4) | 81 | 6 | Yes |
| 12 | (2,3) from (2,4) | 27 | 48 | No |
| 13 | (1,5) from (1,4) or (2,5) | 82 | 7 | Yes |
| 14 | (1,3) from (1,4) | 25 | 50 | No |

Region S2 = column 4 (all 5 pixels) + column 5 (all 5 pixels) = 10 pixels.

Region map (R = in S2):

| . | . | . | R | R |
| --- | --- | --- | --- | --- |
| . | . | . | R | R |
| . | . | . | R | R |
| . | . | . | R | R |
| . | . | . | R | R |

Answer (b): S2 region = {(1,4),(2,4),(3,4),(4,4),(5,4),(1,5),(2,5),(3,5),(4,5),(5,5)}.

## Combined result and remarks

| A | A | A | B | B |
| --- | --- | --- | --- | --- |
| A | A | A | B | B |
| A | A | - | B | B |
| A | A | - | B | B |
| A | A | - | B | B |

A = region of S1 (12 pixels), B = region of S2 (10 pixels), "-" = unassigned: (3,3), (4,3), (5,3) with intensities 50, 52, 55.

The unassigned pixels are too bright for S1 (more than 31) and too dark for S2 (less than 67), so they form the boundary between the two regions.

The two regions do not overlap, so there is no conflict between the seeds.

# Question 5: Hospital image-processing case study

Scenario: grayscale medical images with (i) uneven illumination, (ii) overlapping intensity distributions, (iii) weak boundaries. The goal is to find suspicious regions.

## (a) Limitations of global Otsu thresholding

How Otsu works: it computes the histogram, then tries every threshold T and picks the one that maximises the between-class variance (equivalently, minimises the within-class variance). It outputs one threshold for the whole image.

Single threshold under uneven illumination. If one side of the image is brighter, healthy tissue in the bright area can be brighter than suspicious tissue in the dark area. One T cannot be correct everywhere, which gives false positives in bright zones and missed regions in dark ones.

Overlapping distributions. Otsu works best when the histogram is bimodal with a clear valley between two well-separated peaks. When normal and abnormal intensities overlap, the valley disappears or shifts, and the chosen T misclassifies a large number of pixels from both classes.

No spatial information. Otsu looks only at the histogram. It ignores where pixels are, so it cannot use connectivity or shape. The result is scattered pixels and holes, and isolated noise pixels become foreground.

Weak boundaries. Where the edge between regions is gradual, a hard threshold cuts at an arbitrary position, so the boundary is inaccurate.

Bias for unequal class sizes. When the suspicious region is small compared with the background, its contribution to the histogram is tiny, so the threshold tends to move towards the larger class and small lesions are missed.

Sensitivity to noise. Noise widens the histogram peaks and adds to the overlap.

## (b) Region growing vs. region splitting and merging

| Aspect | Region growing | Region splitting and merging |
| --- | --- | --- |
| Direction | Bottom-up: start from seed pixels and add similar neighbours | Top-down then bottom-up: split the image into quadrants until each is homogeneous, then merge similar adjacent regions |
| Seeds | Required (manual or automatic). The result depends strongly on seed position | Not required. Fully automatic |
| Criterion | Pixel-to-region similarity (e.g. ¦I - seed¦ <= T, or distance to region mean) | Homogeneity predicate on a whole block (e.g. standard deviation or max-min below a limit) |
| Boundary quality | Follows the true local boundary at pixel level when the criterion fits | Blocky edges because of the quadtree structure, unless post-processing is added |
| Weak boundaries | Can leak through a weak edge into neighbouring tissue | Block-level tests can miss a thin weak edge inside a block, or split at the wrong place |
| Noise | Sensitive to noise in the seed and in the threshold | More robust, as decisions are based on statistics of whole blocks |
| Illumination | Handles slow illumination change if compared with local or running mean | A fixed predicate can fail under strong shading |
| Cost | Cheap, processes only the region of interest | More expensive, processes the whole image with recursion and merge passes |
| Connectivity | Gives connected regions | Gives connected regions after merging |
| Order dependence | Result can depend on the order pixels are visited | Result depends on how the quadtree is aligned |

## (c) Recommendation

Recommended approach: region growing, preceded by pre-processing, with an adaptive similarity criterion.

Proposed pipeline:

Pre-processing: correct the uneven illumination (background estimation and subtraction, or homomorphic filtering), reduce noise (Gaussian or median filter), and optionally enhance local contrast (CLAHE).

Seed selection: the radiologist places seeds on suspicious areas, or seeds are chosen automatically (e.g. local intensity maxima, or the output of a coarse Otsu or split-and-merge pass).

Growing with an adaptive criterion: compare a candidate pixel with the current region mean (or a local window) instead of a fixed global value, and add a gradient or edge constraint to stop growth at weak boundaries.

Post-processing: morphological opening and closing to remove small holes and spurs, and a size limit to stop leakage.

Justification:

It relies on local similarity and connectivity, not one global threshold, so it copes with uneven illumination where Otsu fails.

Each region is grown from a seed, so it handles overlapping global distributions: only pixels connected to the seed and similar to it are included, even if other parts of the image have the same intensities.

It produces closed, connected regions with pixel-accurate outlines, which suits delineation of lesions.

Seeds let a clinician decide where to look, so it fits medical workflows and keeps a human in the loop.

Computation is cheap enough for routine use.

Residual risk and how to reduce it: weak boundaries can cause leakage, and the result depends on the seed. The edge constraint, a maximum region size and testing several seeds (checking the outputs agree) reduce this. Split-and-merge can be used only as an optional first pass to suggest seeds, because its blocky boundaries are unsuitable for final delineation.