|  | SVKM’s NMIMS Mukesh Patel School of Technology Management & Engineering / School of Technology Management & Engineering |  |  |
| --- | --- | --- | --- |
| Program: B. Tech CE |  | Lab and Workbook | Academic Year- 2026-27 |
| Year:-Third |  | Subject:- Drone Technology | Semester: - Fifth |

Experiment: 8

PART A

(PART A: TO BE REFERRED BY STUDENTS)

Aim:

- To study how to program Pixhawk Flight controller.

- To learn set up for R Channels.

- To program Pixhawk Flight controller connected drone.

Prerequisites: Basic knowledge of UAVs, Experience with radio transmitters and receivers, Basic

programming knowledge

### Learning outcomes:

After completing the lab, students will be able to:

- Understand the architecture and functionality of the Pixhawk Flight Controller.

- Configure and program the Pixhawk using Mission Planner or QGroundControl.

- Set up and calibrate radio channels (R Channels) for communication between transmitter and receiver.

- Successfully program a Pixhawk-connected drone for autonomous or manual flight.

Material Required

- Flight Controller (PX4 or Ardupilot firmware)

- Computer with Mission Planner or QGroundControl installed

- RC Transmitter and Receiver (with at least 6 channels)

- Drone frame and motors

- Power module

- USB cable for programming

- Battery and other drone accessories

Theory:

Pixhawk Flight Controller: Pixhawk is an open-hardware autopilot that runs ArduPilot or PX4 firmware. It reads sensors, estimates the vehicle state, runs the control loops and drives the motors/servos; it can also fly pre-programmed autonomous missions.

| Block | Details (Pixhawk 1) |
| --- | --- |
| Processor | 32-bit STM32F427 Cortex-M4F @ 168 MHz, 256 KB RAM, 2 MB flash |
| Failsafe co-processor | STM32F100 (32-bit) – keeps manual override if main processor fails |
| Sensors | MPU6000 accelerometer/gyro, L3GD20 gyro, LSM303D accel/magnetometer, MS5611 barometer |
| Outputs | 8 MAIN OUT + 6 AUX PWM outputs (motors/servos) |
| Inputs / ports | RC IN (PPM/SBUS), POWER, GPS, TELEM1/2 (radio), USB, CAN, I2C, ADC |
| Firmware | ArduPilot (Copter/Plane/Rover) or PX4; flashed using Mission Planner or QGroundControl |

R (Radio) Channels: each stick or switch on the transmitter is one channel, sent to the receiver as a pulse width between 1000 µs (minimum) and 2000 µs (maximum), with 1500 µs at centre. The standard ArduCopter mapping is CH1 = Roll, CH2 = Pitch, CH3 = Throttle, CH4 = Yaw, CH5 = Flight-mode switch, CH6–CH8 = auxiliary functions. Radio calibration teaches the autopilot the minimum, maximum and trim values of each channel.

Ground control software: Mission Planner (ArduPilot, Windows) or QGroundControl (PX4/ArduPilot, multi-platform) is used for firmware loading, sensor calibration, radio setup, parameter tuning and mission planning.

### Procedure:

#### 1. Setup Pixhawk Flight Controller:

- Install Ground Control Software:

  - Download and install Mission Planner (for ArduPilot firmware) or QGroundControl (for PX4 firmware) on your computer.

  - Connect the Pixhawk flight controller to your computer using a USB cable.

- Load Firmware:

  - Open the ground control software and connect the Pixhawk.

  - Select the appropriate firmware for your drone. Choose between ArduPilot or PX4.

  - Flash the firmware onto the Pixhawk.

- Calibrate Sensors:

  - After flashing the firmware, follow the on-screen instructions to calibrate the following sensors:

    - Accelerometer

    - Gyroscope

    - Compass

    - RC Transmitter

#### 2. Setting up R Channels:

- RC Transmitter Setup:

  - Connect the RC receiver to the Pixhawk using the provided connections (PPM/SBUS).

  - Turn on your RC transmitter and ensure the channels are functioning correctly.

- Mapping R Channels:

  - In Mission Planner/QGroundControl, go to the "Radio Calibration" tab.

  - Move the sticks on the RC transmitter and observe the corresponding channel movements on the screen.

  - Assign each channel to its respective function (e.g., throttle, pitch, roll, yaw).

- Setting Mode Switch:

  - Assign one of the RC channels (usually Channel 5) to the flight mode switch.

  - Configure different flight modes like Stabilize, Altitude Hold, Loiter, and Auto.

#### 3. Programming the Pixhawk Connected Drone:

- Set Parameters for Flight:

  - Go to the "Full Parameter List" or "Full Parameter Tree" in the ground control software.

  - Set basic parameters for drone control, such as:

    - Throttle mid-point

    - Roll, pitch, and yaw sensitivity

    - Failsafe settings

- Assign Autonomous Missions (Optional):

  - In Auto mode, you can set waypoints for the drone to follow.

  - Use the ground control software to define a mission by clicking on the map to add waypoints.

  - Upload the mission to the Pixhawk.

- Test the Drone:

  - After completing the setup, place the drone in a safe area.

  - Arm the drone by using the arming switch or by moving the throttle stick on your RC transmitter.

  - Test the basic manual flight modes first (e.g., Stabilize).

  - Once comfortable, test the autonomous flight mode (if programmed).

| Roll No.: B106 | Name: Krishang Vikram Singh |
| --- | --- |
| Prog/Yr/Sem: B.Tech CE / Third Year / Fifth Sem | Batch: C1 |
| Date of Experiment: 15/10/24 | Date of Submission: 15/10/24 |

PART B

Task to be performed

- Pixhawk Setup & Connection

- Radio Channels (R Channels) Setup & Calibration

- Programming the Pixhawk Flight Controller

- Motor & ESC Testing

Task 1: Pixhawk Setup & Connection

- Installed Mission Planner, connected the Pixhawk through USB and selected the correct COM port (baud 115200) → Connect.

- Initial Setup → Install Firmware: selected ArduCopter and the frame type (Quad X) and flashed the firmware.

- Calibrated the accelerometer (6 orientations), compass (rotate in all axes) and checked the gyroscope/level calibration.

Task 2: Radio Channels (R Channels) Setup & Calibration

- Connected the FS-iA6B receiver to the Pixhawk RC IN port (PPM) or to a serial port (i-BUS) and powered the transmitter.

- Initial Setup → Mandatory Hardware → Radio Calibration → Calibrate Radio: moved all sticks and switches to their extremes and clicked Done; the green bars showed minimum/maximum for each channel.

- Verified the mapping – Roll (CH1), Pitch (CH2), Throttle (CH3), Yaw (CH4) – and reversed a channel if the bar moved opposite to the stick.

- Flight Modes: set CH5 as the mode switch and assigned Stabilize, Alt Hold, Loiter, Auto, RTL and Land to the six PWM ranges (FLTMODE1–FLTMODE6).

| Channel | Function | PWM range (µs) |
| --- | --- | --- |
| CH1 | Roll | 1000 – 2000 |
| CH2 | Pitch | 1000 – 2000 |
| CH3 | Throttle | 1000 – 2000 |
| CH4 | Yaw | 1000 – 2000 |
| CH5 | Flight mode switch | 1000 – 2000 (6 mode bands) |
| CH6–8 | Auxiliary (tuning, camera, etc.) | 1000 – 2000 |

Task 3: Programming the Pixhawk Flight Controller

| Parameter | Purpose / value |
| --- | --- |
| FRAME_CLASS / FRAME_TYPE | Frame selection – Quad, X |
| THR_MID / MOT_THST_HOVER | Throttle mid-point/hover throttle (≈ 500 or 0.3–0.4) |
| ATC_ANG_RLL_P, ATC_ANG_PIT_P, ATC_RAT_* | Roll/pitch/yaw sensitivity and PID gains |
| FS_THR_ENABLE | Radio failsafe action (e.g. 1 = RTL) |
| BATT_MONITOR, BATT_FS_LOW_ACT | Battery monitor and low-battery failsafe |
| ARMING_CHECK | Pre-arm checks enabled (1 = all) |

- Opened Config/Tuning → Full Parameter List, searched for the parameters above, changed values and clicked Write Params.

- Created a simple mission in Flight Plan (Takeoff → waypoints → RTL) and uploaded it with Write WPs for AUTO mode.

Task 4: Motor & ESC Testing

- Removed all propellers. Performed ESC calibration (all-at-once method) so that every ESC learns the same minimum and maximum throttle.

- Initial Setup → Optional Hardware → Motor Test: tested motors A, B, C, D at about 5–10 % throttle for a few seconds each and checked rotation direction and order (A = Motor 1 … D = Motor 4). Swapped two motor wires where the direction was wrong.

- Armed in Stabilize mode (propellers still off), raised the throttle slowly and confirmed all four motors respond equally; then disarmed.

Safety: always remove propellers during configuration and motor tests.

Observations and Learning:

- The Pixhawk is connected to the PC by USB; Mission Planner flashes the firmware and calibrates the accelerometer, compass and radio.

- Radio calibration shows each channel moving between 1000 and 2000 µs; mapping CH1–CH4 and a mode switch on CH5 is required for correct control and for switching between Stabilize, Alt Hold, Loiter and Auto.

- Parameters such as THR_MID, failsafe and sensitivity settings change how the drone responds; failsafes must be enabled before flight.

- Motor test confirmed correct motor order/direction and that all ESCs are calibrated equally.

- Manual mode was tested first and autonomous mission only after manual flight was stable.

Conclusion:

The architecture and functionality of the Pixhawk flight controller were understood. The Pixhawk was configured and programmed with Mission Planner (firmware, sensor calibration, parameters), the R channels of the FlySky transmitter and receiver were set up and calibrated including the flight-mode switch, and a Pixhawk-connected drone was prepared for manual and autonomous flight after motor and ESC testing. All four learning outcomes were attained.